Cambrai Covers Measuring Instructions.

Here at Cambrai Covers we go to great lengths to ensure that your cover fits your aircraft perfectly.  It is of the utmost importance that any protrusions on the aircraft such as Aerials, Probes, Pitot Tubes and Grab Handles have the proper allowances made for them.

The back of order form shows a plan view of marking the positions of aerials and probes fitted to your aircraft.  We will need to know the positions of anything mounted to the upper exterior of the aircraft (belly mounted items are not usually a problem).  Grab handles are usually already on our patterns as they rarely move.  If you run out of space please use a plain piece of paper to sketch a diagram - it does not have to be a work of art, as it is the dimensions that are important!

Our datum point is the Top Edge of the Windscreen (TEW/s)- where the Perspex meets the metal frame - and the centreline (C/L) of the aircraft - this gives a definitive point.

Always use FROM…TO  measurements (i.e “VHF –10cm to 17cm x 7 cm Stbd. of C/L”).

1.  Measure the Outside Air Temperature Probe (OAT) position if in the windscreen FORWARD of the TEW/s and then left or right of the centreline.  If the OAT is in a different position, it will most likely be on our pattern. Please be certain to indicate this on your order form.

Measure the positions of the aerial bases using the Top Edge Windscreen (TEW/s) as the Zero datum. Show their position relative to this datum point and the centreline, measured to the centreline of the base, and the type of aerial. The common types are listed below:

Types of Aerials.

VHF – Usually take one of two forms.

1.  Base and Whip type – measure the base as above and note the length of the whip.

2.  Blade types – measure the blade base as above and note maximum width of blade.

ADF.  There are two type of ADF antenna – the traditional insulator and long wire to fin type, and the solid state combined loop and sense aerial.

We only need to know the position of the insulator for the traditional type of antenna.  Measure the position of the insulator in the usual way.

The solid state type is usually belly mounted and therefore will not be affected by the cover.

Glideslope Aerials usually take the form of a ‘V’ or ‘Boomerang’.

Small types – often found on Piper aircraft – can be closed under a cover snood (hat).

Large types – often found on Beechcraft aircraft have a collar type closeout.

Combined with ADF types have the ADF sense wire attached to the rear of the base. Measure this type as though it were two separate aerials.

ALSO Measure to the forward-most point on the aerial and note the length of each ‘Arm’ of the aerial and the distance between the widest point of the two ‘Arms’. Please include a rough sketch.

GPS Aerials.

1.  ‘Shark Fin’ (‘Garmin’) type.  Measure to the front of the base and note the length of the base and the maximum height of the aerial.  A snood is fitted for this type of aerial.

2. Solid state ‘Flat’ types.  Measure the aerial in the usual way and note that it is a ‘FLAT type aerial. We fit reinforcement for this type of aerial.

On the reverse of this page is a diagram showing the above method and space to record the positions of your aerial fit. This data can then be transferred to your order form.

Port

OAT PROBE

## ADF on C/L 						 6cm down x 3cm



C/L-----------------------------------+ 8cm         +--------5cm--------|			 	X   Port of C/L

TEW/s---front of a/c

+ 17cm                             + 10cm------------------------------------|

# VHF - 7cm Stbd of C/L 

Tape measure.)--|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|---|

17 16  15 14  13 12  11  10   9    8   7   6    5   4    3    2    1    0   1   2   3    4    5   6   7   8    9

Starboard

PLEASE NOTE : Take EACH measurements from the Top Edge of the Windscreen – not from aerial to aerial!

Measure with the tape flat against the surface of the airframe or Perspex.

ROOF							{---WINDSCREEN---}

TOP (DATUM)			BASE		COWL     PORT

ALL MEASUREMENTS IN ()  INCHES 		CENTIMETRES 					STARBOARD